WEEK 13  OF  13

# Testing, Performance Optimization & Final Submission

Phase: Testing & Submission

| Report Week | Details |
| --- | --- |
| Week Number | Week 13 of 13 |
| Project Phase | Testing & Submission |
| Objective | Conduct thorough testing across devices and environments, resolve remaining issues, and prepare all final deliverables. |
| Key Deliverable | Final release APK, complete documentation package, demonstration video, and presentation slides |
| Status | ✔ Completed |

ACTIVITIES PERFORMED

- Tested application on multiple Android devices with varying hardware specifications and screen sizes

- Conducted environment testing: indoor (corridors, rooms, offices) and outdoor (roads, campus, open areas)

- Tested under varied lighting conditions: bright daylight, overcast, low light, and artificial indoor lighting

- Optimized frame processing pipeline to maintain stable inference FPS across all tested devices

- Fixed memory management issues — resolved image buffer accumulation causing gradual performance degradation

- Validated audio feedback accuracy against live detection output across all 7 classes

- Generated final release APK build

- Prepared complete project documentation, FYP final report, demonstration video, and presentation slides

ISSUES ENCOUNTERED & RESOLUTIONS

Issue 1:  Application crashed on a lower-end Android device due to insufficient RAM during model inference — the 640×640 input resolution exceeded available memory.

Resolution:  Reduced inference input resolution to 320×320 for low-memory devices, which maintained acceptable detection quality while significantly reducing memory footprint and eliminating the crashes.

Issue 2:  In direct bright sunlight, camera auto-exposure caused frames to become severely overexposed, significantly degrading detection quality in outdoor conditions.

Resolution:  Implemented partial mitigation by locking camera exposure to a fixed value in code to reduce extreme overexposure. This limitation is acknowledged and documented in the final project report as a known constraint for future improvement.